Blood pressure measurement the blood pressure is measured by either a direct or an indirect technique. The direct method requires an invasive procedure in which an intravenous catheter with an electronic sensor is inserted into an artery and the artery-transmitted pressure on an electronic display unit is read. The indirect method requires use of the sphygmomanometer and stethoscope for auscultation and palpation as needed  .

Sites

The most common site for indirect blood pressure measurement is the client’s arm over the brachial artery. When the client’s condition prevents auscultation of the brachial artery, the nurse should assess the blood pressure in the forearm or leg sites .

Type of sphygmomanometer :-

- Mercury

- Air type

- Electronic type

Contraindications for brachial artery blood pressure measurement :-

When the client has any of the following, do not measure blood pressure on the involved arm

- Venous access devices, such as an intravenous infusion or arteriovenous fistula for renal dialysis

- Surgery involving the, axilla, shoulder, arm, or hand

- Injury or disease in the shoulder, arm, or hand, such as trauma, burns

- Application of a cast or bandage .

When pressure measurements in the upper extremities are not accessible, the popliteal artery, located behind the knee, becomes the site of choice. The nurse can also assess the blood pressure in other sites, such as the radial artery in the forearm and the posterior tibial or dorsalis pedis artery in the lower leg. Because it is difficult to auscultate sounds over the radial, tibial, and dorsalis pedis arteries, these sites are usually palpated to obtain a systolic reading.

Before starting any method of tacking B.P  ( by using mercury or air type ). you must be done .

| Action | Rationale |
| --- | --- |
| - Explain to pt. the vital signs will be assessed | To make psychological support , allays anxiety and to encourage pt. participate |
| - Compress pump to inflate cuff | To determined air leak during inflation |
| - Determine which extremity is most appropriate for measuring . Do not take a pressure reading on an injured or painful extremity or one in which an intravenous line is running | Cuff inflation can temporarily interrupt blood flow |
| - Select a cuff size that completely encircles upper arm without overlapping | Provides equalization of pressure on the artery to ensure accurate measurement |
| - Assess pt's toileting needs and proceed as appropriate | Prevent interruptions during ,communicates caring and promotes pt comfort |
| - Provide for privacy | Decrease embarrassment |
| - Wash hand & apply gloves | Reduce transmission of microorganism |

Procedure :- ( using mercury or air type Sphygmomanometer)

| Action | Rationale |
| --- | --- |
| 1- Move clothing away from upper aspect of arm | Ensures accurate measurement |
| 2- Position arm at heart level, extend elbow with palm turned upward | Blood pressure increases when arm is below level of heart and decreases when arm is above level of heart. |
| 3- Make sure cuff is fully deflated and pump valve moves freely | Equipment must function properly to obtain an accurate reading |
| 4- Palpate brachial artery | placement of stethoscope |
| 5- Apply cuff snugly and smoothly over upper arm, 2.5 cm above antecubital space | Ensures even pressure distribution over brachial artery |
| 6- turn valve clockwise to close and compress bulb to inflate cuff until where palpated pulse disappears, then slowly release valve (deflating cuff), noting reading when pulse is felt again | Inflates the cuff with pressure and temporarily impairs flow of blood through artery. Provides an estimate of maximum pressure required to measure systolic pressure |
| 7- Insert earpiece of stethoscope in ears with a forward tilt, ensuring diaphragm hangs freely | Enhances sound transmission from bell to ears. |
| 8- Relocate brachial pulse with your hand and place bell directly over pulse. bell should be in direct contact with skin and not touch cuff | Sound heard best directly over artery; decreased Other sounds that cause inaccurate reading |
| 9- With other hand, turn valve clockwise to close. Compress pump to inflate cuff until (number) of mm hg in which diminished pulse point identified ( in step 6) | Ensures the cuff is inflated to a pressure greater than the client’s systolic pressure. |
| 10-Slowly turn valve counterclockwise so that mercury falls at a rate of 2–3 mm Hg per second. Listen to first sound & last sound | First sound is a systolic pressure & last sound is diastolic pressure . |
| 11- Deflate cuff rapidly and completely | Prevents arterial occlusion and client discomfort of numbness or tingling. |
| 12- Remove cuff or wait 2 minutes before taking a second reading | Releases trapped blood in the vessels |
| 13- Inform client of reading | Promotes client participation in care |
| 14-Dispose gloves & wash hand | Reduce transmission of microorganisms |
| 15- Record reading | Ensures accuracy |
| 16- Wash hand | Reduce transmission of microorganisms |

Using electronic type

Before starting any method of tacking B.P  ( by using mercury or air type ). you must be done .

| Action | Rationale |
| --- | --- |
| - Explain to pt. the vital signs will be assessed | To make psychological support , allays anxiety and to encourage pt. participate |
| - Determine which extremity is most appropriate for reading. Do not take a pressure reading on an injured or painful extremity or one in which an intravenous line is running | Cuff inflation can temporarily interrupt blood flow |
| - Select a cuff size that completely encircles upper arm without overlapping | Provides equalization of pressure on the artery to ensure accurate measurement |
| - Assess pt's toileting needs and proceed as appropriate | Prevent interruptions during ,communicates caring and promotes pt comfort |
| - Provide for privacy | Decrease embarrassment |
| - Wash hand & apply gloves | Reduce transmission of microorganism |

Procedure :-

| Action | Rationale |
| --- | --- |
| 1- Move clothing away from upper aspect of arm | Ensures accurate measurement |
| 2- Position arm at heart level, extend elbow with palm turned upward | Blood pressure increases when arm is below level of heart and decreases when arm is above level of heart. |
| 3- Make sure cuff is fully deflated | Equipment must function properly to obtain an accurate reading |
| 4- Apply cuff snugly and smoothly over upper arm, 2.5 cm above antecubital space | Ensures even pressure distribution over brachial artery |
| 5-Press on start or turn and wait some of seconds . | To start play on . the reading of B.P need some of seconds to appear |
| 6- Turn off electronic sphygmomanometer | To prepare it for next procedure |
| 7-Remove cuff or wait 2 minutes before taking a second reading | Releases trapped blood in the vessels |
| 8-Inform client of reading | Promotes client participation in care |
| 9-Dispose gloves & wash hand | Reduce transmission of microorganisms |
| 10-Record reading | Ensures accuracy |
| 11-Wash hand | Reduce transmission of microorganisms |